RPSDM V4 Design Decisions

PRE BUILD CAD WALKTHROUGH VIDEO. (Video does not represent the final version as some changes were made.)

- Colour Code Fasteners

  - Passive

  - Interactive

- NATO green paint (Available everywhere in world)

- Deck Stength

  - Thicker nose and tail

  - Thicker in the middle (T Beam)

  - Steep concave (Pipe)

  - Thick patch on nose and tail curve up (Prevents wearring)

- Springs to allow nozzles to move slightly when coupling with the nozzle plate

- Hold thermocouple

  - Nut welded in beam

- Differentiate nose/tail

  - Arrow on horizontal plate

- Change Deck Thickness

  - ‘Washer” Plates

- Attach to Extruder

  - Quick Release: Big Laser cut handle welded to nut

  - Find right plumbing head: Swivel connector

- Open Mould

  - Nuts on top and bottom plate for “Release Bolts”

  - Enough gap between side frame and horizontal plate to fit in crowbar

- Heat Mould

  - Cartridge heaters (380, 16A) in holes down side of “Deck Edge”

- Fit aluminium sections in

  - Fillet inside gradual

  - 0.2mm contour between deck top/bottom face and deck edge (Check V4 file)

- Slow to bolt mould together

  - Drill with long socket

  - Countersink holes on steel plate

- Mark Holes For Trucks

  - 5mm Dowel inserts in mould

- Stop Mould Bend

  - C beams (Out)

  - 80x40 frame on top (C Beams top also??) Then all C Beams

Part Numbers Template

before the dot is only for assemblies, assemblies always end in .00

X000.00 - in this case X is for upper level assemblies

0X00.00 - in this case X is for sub assemblies

00X0.00 - in this case X is for sub sub assemblies and so on

0000.xx After the dot is the part number

EXTPRO-V1 0000.00

| EXTPRO- | V1 |  | 0 | 0 | 0 | 0 | . | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Machine Name | V# |  | Part | Assembly | Sub Assembly |  |  |  | Part |
| RPSDM- | V3 |  | -Deck -Mould -Electronics Enclosure -Table | -Mould Top -Mould Bottom | -Mold Top Support Frame |  |  |  | Bolt Beam |